Assignment 1: Serial Optimisations

Andrew Smith (nj19451)

Compilers
A good worker uses good tools, so it is prudent to first experiment with compilers and their optimisation flags, to see if there are runtime gains to be made.  
Figure 1 shows both gcc and icc compiled runtimes, "1024 1024 100" as arguments "nx ny niters".  The ‘Ofast’ setting makes considerable gains in runtime, gcc’s running 5 times faster than without optimisation, and icc’s ‘Ofast’ was 23 times as fast.  However, the icc ‘fast’ flag was fastest of all by far (0.18s) over 31 times faster.  The ‘native’ flag did not result in more efficient runtime, this because the compilation location is different to the runtime location.  Obviously, the native location to which the code is tuned is not in fact used at runtime.

The order of the compiler flags matters as well so ‘fast’ (which uses the flag ‘xHost’) being the last flag invoked would override any previous relevant flags and so 'native' was invoked last.  The issue could be overcome by use of the ‘xCORE-AVX2’ flag which is the correct flag (Intel, 2019) for Broadwell CPUs used by BlueCrystal Phase 4 (UoB, 2017).  A simpler method is just to use the ‘fast’ flag which implements the highest  instruction set available (Intel, n.d.) (on compilation node) and was clearly faster anyway.

Data_Structures
Since large data types possess more bytes, less data can be actioned upon at a time.  This may well significantly impact runtime – since the bottleneck of stencil is memory bandwidth.  This is implied by a count of the assembly occurrence of 'mov' versus arithmetic operations (add, xor, mul) shows there’s twice as many load/store operations.  'image' and 'tmp_image' and others in the function stencil, were used in experimentation of data sizes with the intention of making better use of cache.
Operational Intensity=9 ops40 bytes

Since the original stencil code initialised stencil elements as double it was attempted with floats instead.  The results are shown in Figure 2 as ‘None’ (re tiling).

Figure 1: Comparison of compilers: gcc Vs icc

Vectorization
Efficient vectorisation requires program data to be aligned, in this case at least by due to the size of the cache line, 64-bytes.  Without alignment, extra instructions are required before and after the aligned blocks which reduces the ratio of non-vector instructions to SIMD.  The pointers ‘image’ and ‘tmp_image’ were malloc’d with ‘_mm_malloc’ which ensures the data allocated is aligned to a parameter given in the function invocation.

In function stencil, ‘__assume’ and ‘__assume_aligned’ were used to signal to the compiler the data was safe to vectorise.  With these additions and ‘#pragma ivdep’ the compiler can be persuaded to produce SIMD instructions for the for loop which resulted in an average of 0.22 s runtime – 26x faster than non-optimised.  However, since the ‘fast’ flag produces vectorising instructions and is yet faster it seemed best to just use that.

The vector reports highlight that despite the aligned memory for ‘image’ and ‘tmp_image’ there is unaligned access because of the innermost loop arithmetic.  Without changes to the algorithm this is unavoidable.

Tiling

By tiling the data, the data should be reused with fewer transfers from processor/cache to memory.  This transferral is slow compared to processor/cache transfer times (1/4 cycles compared to 200+ cycles (Deakin, 2019)) So, it is better to reuse data already in cache than having to retrieve the same data repeatedly.  The data was initialised by tiling so a natural tile size to try would be the same.

Tiling resulted in runtimes around the 1 s mark, but the vector reports highlighted "Loop Interchange" which implied changing the iteration variables might improve vectorisation.  With the variables i and j swapped, the runtime decreased 10-fold.  Figure 2 shows the results of the investigation into different tile sizes, run with "icc -fast”.  As shown, increasingly large tile sizes reduced the runtime.

Other Serial Optimisation Strategies

Algorithm re-organisation

Since there is index arithmetic in the innermost for loop, full vectorisation is difficult.  This is apparent in the speed differences in output/init functions and stencil function.  Moreover, since the data is unaligned, it makes sense to align it.

Analysis of the indices used in the cell blurring algorithm shows that each cell called (corresponding to a load operation) is called up to five times.  These five calls happen because of the five lines with arithmetic index differences in the inner loop.  Considering this feed forwarding (Waidyasooriya, et al., 2017) was attempted.  Because the algorithm is memory bandwidth limited, it was assumed additional operations within the inner loop might be a small price to pay for better utilisation of data – while it is in a higher level of cache.

Threading

An intriguing option would be to make use of threading.  This would abide by the single processor rule while potentially drastically increasing runtime by computing two different threads.  A functioning prototype was quickly developed but the runtime reductions were not seen.  Unfortunately, this was not investigated as thoroughly as might necessitate due to time constraints.

Conclusion

There are many serial optimisations to apply before parallelisation.  The choice of tool in this report bringing the most drastic runtimes down.  Switch from gcc to icc for 3.3 times faster runtime.  Invoke the 'fast' flag on icc for 31 times faster than the gcc original.

Experiments in Figure2 showed that halving data type size overhead halved runtime, this being 60 times faster than zero optimisation choices.  The effect of tiling could not match this until tile size of 1024.  This larger tile size paid dividends, however, in the larger stencil input sizes, especially 8000.

1024 B

512 B

256B

128 B

64 B

32 B

None

References

Figure 2: Tile size experimentation

Deakin, T., 2019. Introduction to High Performance Computing (COMS30005) 'Serial Optimisations' Lecture. Bristol, UoB.

Intel, 2019. Intel® Xeon® Processor E5-2600 v4 Product Family. [Online] 
Available at: https://software.intel.com/en-us/articles/intel-xeon-processor-e5-2600-v4-product-family-technical-overview
[Accessed 18 10 2019].

Intel, n.d. xHost, QxHost. [Online] 
Available at: https://software.intel.com/en-us/cpp-compiler-developer-guide-and-reference-xhost-qxhost
[Accessed 15 10 2019].

UoB, 2017. BlueCrystal Phase 4. [Online] 
Available at: https://www.acrc.bris.ac.uk/acrc/phase4.htm
[Accessed 15 10 2019].

Waidyasooriya, H., Takei, Y., Tatsumi, S. & Hariyama, M., 2017. OpenCL-Based FPGA-Platform for StencilComputation and Its Optimization Methodology. IEEE TRANSACTIONS ON PARALLEL AND DISTRIBUTED SYSTEMS, 28(5), pp. 1390 - 1402.